Instruments de mesure verticaux

Height Gauges

CONTENTS

|  | TVM 5 |
| --- | --- |
|  | V3 & V4 11 |
|  | V5 & V6 17 |
|  | V7 23 |
|  | V8 29 |
|  | V9 35 |
|  | Accessories 45 |

PRESENTATION

Founded more than 45 years ago by metrology enthusiasts, Trimos has always kept a pioneering spirit by offering innovative solutions at the cutting edge of technology. Dimensional metrology is part of our DNA. Trimos' core business is focused on the development, manufacture and marketing of dimensional measuring instruments as well as all tools for their efficient operation (accessories, software, maintenance). The product palette includes: - Height gauges - Length measuring and calibration instruments - Optical instruments for surface measurement Our philosophy is to offer products and solutions that help our customers improve their productivity. We achieve this goal by respecting 3 fundamental principles: Simplicity of use thanks to easy to understand interfaces, reliability by the use of proven components and precision by integrating the best measurement systems. Resolutely oriented towards the future, the latest generation of Trimos instruments is ready to face the challenges of the next industrial revolution.

Patrice Kemper, CEO

TRIMOS SA

Av.de Longemalle 5

CH - 1020 Renens

www.trimos.com

QUALITY

Quality has always been Trimos' main concern. Our organization, certified ISO9001 for many years, aims to offer its customers excellent products and services. All instruments are developed and produced in our workshops by highly qualified personnel. We thus fully control the quality. By choosing a Trimos instrument, you benefit from more than 45 years of experience in the field of metrology. This is the guarantee of a high-end instrument bearing the "Swiss Made" label.

LABORATORY

In order to guarantee the highest performance of its instruments, Trimos has a calibration laboratory equipped with the latest technologies. Environmental conditions are monitored using the exclusive Trimos® Environment Control System. Thus, a seamless monitoring of temperature, humidity and pressure is guaranteed 24 hours a day.

SERVICES

| Standard measuring instruments | The true essence of our activity is to develop, manufacture and market dimensional measuring instruments. |
| --- | --- |
| Technical support | Our team of specialists is at your disposal to help you find a solution to your metrological problems. |
| Training | We offer training to exploit the full potential of your measuring instruments, on site or in our premises. |
| Consulting in metrology | We help you plan your lab so you can optimize measurement times and minimize uncertainties. |
| Repair | Trimos instruments last a long, long time! We repair them well beyond the deadlines prescribed by law. |
| Calibration | Our laboratory offers a fast and efficient calibration service for all Trimos instruments regardless of their generation. |
| Customized solutions | Our engineering department develops metrological solutions (mechanical, electronic, software) tailored for your applications. |
| Maintenance contracts | Thanks to our network of global agents we are able to offer you maintenance contracts adapted to your needs. |

WEBSITE

The website www.trimos.com is at your disposal. You will find the latest information on our company, products and our sales network. A dedicated website for height gauges was also released recently: www.height-gauge.com Do not hesitate to contact us!

TVM

Introduction

The TVM instrument range is ideal for height measurement and scribing applications in a production facility. Its unique design makes it very mobile, compact and extremely robust. Thanks to an advanced low energy measuring system, its autonomy is about a year. Its ease of use allows you to minimize user training time and use it in multi-station. Its success is absolutely unmatched in the world of the vertical column, more than 16'000 units sold. We offer a complete range of instruments ranging from 300 mm to 600 mm, as well as accessories to meet all your needs.

Robust and compact design

Perfect in workshop environment

Accurate and reliable measurements

Easy handling

Opto-RS232 interface

AUtonomous use on battery

TWO VARIANTS: CAST IRON OR GRANITE BASE

dESCRIPTION

Heights

Diameters

Centerlines

Min / Max / Delta

Handwheel for the displacement with fine adjustment Cast iron base of functional design or granite plate (on request) Digital display Column made of hard chromium plated steel, precision ground Interchangeables accessories 2 references

Display/Software

The clearly defined functions of the display unit allow to collect all height measuring values.

Min/Max function indicator

2 references

End of battery life warning indicator

Preset selection

Measuring unit (mm / in )

Tolerance mode

Technical Specifications

| TVM | 304 | 604 |
| --- | --- | --- |
| Measuring range mm (in) | 320 (12) | 620 (24) |
| Max. permissible errors 1) mm | 0.02 | 0.03 |
| Repeatability (2s) 1) mm | 0.005 |  |
| Frontal squareness deviation mm | 0.02 | 0.03 |
| Resolutions mm (in) | 0.01 / 0.001 (.0005/.00005) |  |
| Measuring force N | 3 |  |
| Autonomy h | 2000 |  |
| Data output | Opto RS232 |  |
| Weight (cast iron base) kg | 6 | 10 |
| Weight (granite base) kg | 14 | 18 |

The above values have been determined according to ISO 13225 with the standard insert (TVM1 / 1.1 / 2).

Diagram

Cast iron base	Granite base

STANDARD INSTRUMENT

| The TVM instruments are supplied as follows |  |
| --- | --- |
| Instrument according to specifications | Protection cover (TVM.HO300/600) |
| Measuring insert and holder (TVM1/1.1/2) | User’s manual (750 50 0018 03) |
| Lithium battery, 3 V (BAT-TVM.OPTO) | Calibration certificate |

Order numbers

| TVM Cast iron base | TVM Granite base |  |
| --- | --- | --- |
| TVM304 700 102 10 11 | TVM304G 700 102 10 14 | Measuring range 300 mm |
| TVM604 700 102 20 11 | TVM604G 700 102 20 14 | Measuring range 600 mm |

applications

| Height and depth measurements (TVM1/1.1/2) | Diameter measurement with bidirectional probe (TVM4) | Centreline measurement with cones (TVM5) |
| --- | --- | --- |
| Scribing (TVM1/TVM3) | Easy handling |  |

V3 & V4

Introduction

Vertical measuring instruments V3 and V4 have been developed for the most severe shop environments. Made entirely in Switzerland, they offer their users the guarantee of a robust, precise and reliable product in the long term. The philosophy of Trimos for over 40 years is to offer simple and highly effective instruments in the production environment. The display unit is provided with functions that are directly accessible and represented by easily understandable symbols. They allow a quick start, even by unqualified staff. The large 2-line "Black Mask" display offers exceptional contrast in all lighting conditions, a unique feature on the market. V3 is the entry level of Trimos universal measuring height gauges. It benefits fully from developments on the higher models. Robust in design and easy to use, it concentrates the essential functions for use in the workshop. The V4 offers the same functions as the V3. It is also equipped with a second key holder and an air cushion for easy movement on the measuring table. The programmable function buttons on the handle allow quick access to the most used functions.

Measuring ranges 400 AND 700 MM

EXTREMELY SIMPLE USE

ELECTRONICALLY ADJUSTABLE MEASURING FORCE

TEMPERATURE COMPENSATION

Standard inserts up to 400 mm

VAST RANGE OF ACCESSORIES

ALL POSSIBLE ADJUSTMENTS WITHOUT TOOLS

RS232 AND USB INTERFACES

| Heights |  |
| --- | --- |
| Diameters |  |
| Centreline distances |  |
| Min / Max / Delta |  |
| 9 references |  |
| Perpendicularity |  |
| Average |  |
| Difference |  |
| Temperature comp. |  |
| Wireless communication (V4) |  |

dESCRIPTION

Handwheel for carriage displacement (manual)

Cast iron base for optimum stability

Interchangeable insert and insert holder

Upper probe holder (V4)

Floating probe suspension adjustment

"Black Mask" display with intuitive function keys

Handle for instrument displacement with

programmable function keys and air cushion button (V4)

Display/Software

Keyboard functions are represented by clear and intuitive symbols. The display on 2 lines offers the user a great working comfort.

EXCELLENT CONTRAST WITH "BLACK MASK" DISPLAY

MEASUREMENTS OF HEIGHTS OR DIAMETERS

MIN / MAX / DELTA MEASURING MODES

ZERO OR PRESET

USB AND RS232 DATA OUTPUT

9 REFERENCES AVAILABLE

GRAPHICal AND acoustic INDICATOR

SMART REVERSE DIAMETER MEASUREMENT

Technical Specifications

| V3 | 400 | 700 |
| --- | --- | --- |
| Measuring range mm (in) | 407 (16) | 711 (28) |
| Application range mm (in) | 508 (20) | 812 (32) |
| Max. permissible errors, BMPE µm | 7 | 8 |
| Repeatability, RMPE (2s) µm | 2 (Ø: 4) |  |
| Perpendicularity deviation (frontal), SMPE µm | 10 | 15 |
| Max. resolution mm (in) | 0.001 (0.00005) |  |
| Measuring force N | 0.75 ÷ 1.5 |  |
| Autonomy h | 40 |  |
| Data output | USB / RS232 |  |
| Air cushion | No |  |
| Weight kg | 21 | 24 |

| V4 | 400 | 700 |
| --- | --- | --- |
| Measuring range mm (in) | 407 (16) | 711 (28) |
| Application range mm (in) | 719 (28) | 1023 (40) |
| Max. permissible errors, BMPE µm | 4.5 | 6 |
| Repeatability, RMPE (2s) µm | 2 (Ø: 4) |  |
| Perpendicularity deviation (frontal), SMPE µm | 10 | 15 |
| Max. resolution mm (in) | 0.001 (0.00005) |  |
| Measuring force N | 0.75 ÷ 1.5 |  |
| Autonomy h | 20 |  |
| Data output | USB / RS232 / Wireless (optional) |  |
| Air cushion | Yes |  |
| Weight kg | 21 | 24 |

The above values have been determined according to ISO 13225 with the standard insert (TA-MI-101)

Diagram

L: depends on the measuring insert used

STANDARD INSTRUMENT

| The V3 and V4 are supplied as follows |  |
| --- | --- |
| Instrument according to specifications | Charging unit (TA-EL-132) |
| Measuring insert with ruby ball Ø 4 mm (TA-MI-101) | Setting gauge (TA-MG-104) |
| V3: Quick guide (750 50 0046 00) | V4: User's manual (750 50 0045 03) |
| Calibration certificate |  |

Order numbers

| V3 | V4 |  |
| --- | --- | --- |
| V3-400 700 110 10 03 | V4-400 700 110 10 04 | Measuring range 400 mm |
| V3-700 700 110 20 03 | V4-700 700 110 20 04 | Measuring range 700 mm |

applications

| Measurements of height, thickness and chain of dimensions (TA-IH-135, TA-MI-101) | Simultaneous display of diameter and centreline (TA-IH-135, TA-MI-101) | All instruments are mechanically corrected (TA-IH-135) |
| --- | --- | --- |
| Standard measuring inserts up to 400 mm, with excellent repeatability (TA-IH-131, TA-AD-105, TA-IH-115, V-50.4) | Very large range of accessories for any type of application (TA-SE-106, TA-SE-102, TA-SE-105, TA-SE-107) | Straightforward symbols and directly accessible functions Display with maximum contrast |

|  | SMART REVERSE Diameter measurement faster, more accurate and simpler SmartReverse technology is the result of an intense collaboration between Trimos height gauge users and our R&D team in order to optimize diameter measurements. SmartReverse makes the measurement of diameters very effective by clearly indicating the passage of the reversal points by audible and visual signals. The user is guided precisely during the measurement of diameters, which generates a significant gain in speed and reliability of the measurement. |
| --- | --- |

V5 & V6

Introduction

The V5 and V6 come from the legendary V + height gauge family, which has been sold thousands of times in workshops worldwide and universally recognized for their ease of use and quality. Trimos was the first to offer a vertical measuring instruments on the market more than 40 years ago. The V5 and V6 represent a concentrate of know-how accumulated during all these years. The easy-to-read display and intuitive functions offer exceptional user comfort. But that's not all: The V5 and V6 are equipped with a revolutionary displacement crank offering the user the choice of how to move the measuring carriage, either manual or motorized. Each of these modes does not suffer from any compromise, that is to say that the user preferring a manual instrument will note no difference compared to a conventional manual instrument, ditto for motorized movement. This innovation avoids making a difficult choice during the acquisition and makes it possible to satisfy the multiple potential users of the same instrument. The design of the V5 with its lateral probe holders revives an old tradition of Trimos: Enable measurement with long sturdy keys while ensuring excellent repeatability. The V6 is of the same calibre as the V5. It is equipped with a more precise measuring system and allows the use of an electronic probe of perpendicularity.

Measuring ranges 400, 700 AND 1100 MM

EXTREMELY SIMPLE USE

ELECTRONICALLY ADJUSTABLE MEASURING FORCE

MANUAL OR MOTORIZED DISPLACEMENT

STANDARD inserts UP TO 400 MM

VAST RANGE OF ACCESSORIES

ALL POSSIBLE ADJUSTMENTS WITHOUT TOOLS

RS232 AND USB INTERFACES

| Heights |  |
| --- | --- |
| Diameters |  |
| Centreline distances |  |
| Min / Max / Delta |  |
| 9 references |  |
| Perpendicularity |  |
| Average |  |
| Difference |  |
| Temperature comp. |  |
| Wireless communication |  |

dESCRIPTION

Handwheel for carriage displacement (manual/motorized)

Cast iron base for optimum stability

Interchangeable insert and insert holder

Upper probe holder

Floating probe suspension adjustment

"Black Mask" display with intuitive function keys

Handle for instrument displacement with

programmable function keys and air cushion button

Display/Software

Keyboard functions are represented by clear and intuitive symbols. The display on 2 lines offers the user a great working comfort.

EXCELLENT CONTRAST WITH "BLACK MASK" DISPLAY

MEASUREMENTS OF HEIGHTS OR DIAMETERS

MIN / MAX / DELTA MEASURING MODES

ZERO OR PRESET function

USB AND RS232 DATA OUTPUT

9 REFERENCES AVAILABLE

GRAPHIC AND acoustic PROBE INDICATOR

SMART REVERSE DIAMETER MEASUREMENT

Technical Specifications

| V5 | 400 | 700 | 1100 |
| --- | --- | --- | --- |
| Measuring range mm (in) | 407 (16) | 711 (28) | 1110 (44) |
| Application range mm (in) | 719 (28) | 1023 (40) | 1422 (56) |
| Max. permissible errors, BMPE µm | 2.5 + L(mm)/300 |  |  |
| Repeatability, RMPE (2s) µm | 2 |  |  |
| Perpendicularity deviation (frontal), SMPE µm | 5 | 8 | 11 |
| Max. resolution mm (in) | 0.0005 (0.00005) |  |  |
| Measuring force N | 0.75 ÷ 1.5 |  |  |
| Autonomy h | 12 |  |  |
| Data output | USB / RS232 / Wireless (optional) |  |  |
| Air cushion | Yes |  |  |
| Weight kg | 21 | 24 | 33 |

| V6 | 400 | 700 | 1100 |
| --- | --- | --- | --- |
| Measuring range mm (in) | 407 (16) | 711 (28) | 1110 (44) |
| Application range mm (in) | 719 (28) | 1023 (40) | 1422 (56) |
| Max. permissible errors, BMPE µm | 2 + L(mm)/400 |  |  |
| Repeatability, RMPE (2s) µm | 1 (Ø: 2) |  |  |
| Perpendicularity deviation (frontal), SMPE µm | 5 | 8 | 11 |
| Max. resolution mm (in) | 0.0001 (0.000005) |  |  |
| Measuring force N | 0.75 ÷ 1.5 |  |  |
| Autonomy h | 12 |  |  |
| Data output | USB / RS232 / Wireless (optional) |  |  |
| Air cushion | Yes |  |  |
| Weight kg | 21 | 24 | 33 |

The above values have been determined according to ISO 13225, with the standard measuring insert (TA-MI-101).

Diagram

L: depends on the measuring insert used

STANDARD INSTRUMENT

| The V5 and V6 are supplied as follows |  |
| --- | --- |
| Instrument according to specifications | Charging unit (TA-EL-132) |
| Measuring insert with ruby ball Ø 4 mm (TA-MI-101) | Setting gauge (TA-MG-104) |
| Protection cover (TA-TO-114/115/116) | User's manual (750 50 0045 03) |
| Calibration certificate |  |

Order numbers

| V5 | V6 |  |
| --- | --- | --- |
| V5-400 700 110 10 05 | V6-400 700 110 10 06 | Measuring range 400 mm |
| V5-700 700 110 20 05 | V6-700 700 110 20 06 | Measuring range 700 mm |
| V5-1100 700 110 30 05 | V6-1100 700 110 30 06 | Measuring range 1100 mm |

applications

| Measurements of height, thickness and chain of dimensions (TA-IH-135, TA-MI-101) | Simultaneous display of diameter and centreline (TA-IH-135, TA-MI-101) | Measurement of perpendicularity with electronic probe (V6) (TA-IH-136, TA-MS-101) |
| --- | --- | --- |
| Standard measuring inserts up to 400 mm, with excellent repeatability (TA-IH-131, TA-AD-105, TA-IH-115, V-50.4) | Very large range of accessories for any type of application (TA-SE-106, TA-SE-102, TA-SE-105, TA-SE-107) | The instrument can be controlled remotely. This allows its integration into complex quality systems. |
| Manual Motorised |  |  |
| Instant switch from the manual to the motor-driven mode | The motorization guarantees a perfectly constant measuring force (TA-IH-135, TA-MI-101). | Very efficient diameter measurement thanks to SmartReverse technology |

V7

Introduction

The series of V7 height gauges combines technological innovation with tradition. With a full touch display and proven lateral probe holders for decades, the V7s are positioned as universal instruments for the workshop. Although it has been completely redesigned, the interface retains the philosophy of Trimos instruments and the user will have no difficulty in handling it quickly. The touch screen simplifies the use to the maximum because no superfluous information is displayed and the number of function buttons can be limited to the strictest necessary. Functions commonly considered complex such as 2D, programming, statistics become child's play. The result is unmatched ease of use and therefore a significant increase in productivity. The 2 lateral probe holders are from generations of instruments that have forged the reputation of Trimos. Their robustness and flexibility allow the use of very varied inserts up to 400 mm in length with breathtaking repeatability. The V7s are equipped with the revolutionary displacement handwheel allowing the user to choose the manual or motorized movement mode.

Measuring ranges FROM 400 TO 1800 MM

SIMPLE AND CONVIVIAL GRAPHIC INTERFACE

ELECTRONICALLY ADJUSTABLE MEASURING FORCE

MANUAL OR MOTORIZED DISPLACEMENT

2D, PROGRAMMING, STATISTICS

VAST RANGE OF ACCESSORIES

ALL POSSIBLE ADJUSTMENTS WITHOUT TOOLS

RS232 AND USB INTERFACES

| Heights |  |
| --- | --- |
| Diameters |  |
| Centreline distances |  |
| Min / Max / Delta |  |
| 9 references |  |
| Perpendicularity |  |
| Average |  |
| Difference |  |
| Angles and cones |  |
| Calculation function |  |
| Tolerances |  |
| 2D mode |  |
| Programs |  |
| Statistics |  |
| Temperature comp. |  |
| USB ports |  |
| RS232 ports |  |
| Wireless communication |  |

dESCRIPTION

Handwheel for carriage displacement (manual/motorized)

Cast iron base for optimum stability

Interchangeable insert and insert holder

Upper probe holder

Floating probe suspension adjustment

Touch display with intuitive function keys

Handle for instrument displacement with

programmable function keys and air cushion button

Display/Software

The tablet-type display and the graphical interface correspond to the most modern industrial standards. The great flexibility offered by the touch screen allows easy and fast handling. The Smart Reverse function significantly improves diameter measurements.

VERY SIMPLE INTERFACE

GRAPHIC HELP FOR MEASUREMENT

MEASUREMENTS IN 2D MODE

PROGRAMMING MEASUREMENT SEQUENCES

STATISTICAL ANALYSIS OF RESULTS

TEMPERATURE COMPENSATION

SMART REVERSE DIAMETER MEASUREMENT

SMART REVERSE: Diameter measurement faster, more accurate and simpler

|  | SmartReverse technology is the result of an intense collaboration between Trimos height gauge users and our R&D team in order to optimize diameter measurements. SmartReverse makes the measurement of diameters very effective by clearly indicating the passage of the reversal points by audible and visual signals. The user is guided precisely during the measurement of diameters, which generates a significant gain in speed and reliability of the measurement. |
| --- | --- |

Technical Specifications

| V7 | 400 | 700 | 1100 | 1800 |
| --- | --- | --- | --- | --- |
| Measuring range mm (in) | 407 (16) | 711 (28) | 1110 (44) | 1810 (71) |
| Application range mm (in) | 719 (28) | 1023 (40) | 1422 (56) | 2122 (83) |
| Max. permissible errors, BMPE µm | 2 + L(mm)/400 |  |  | 2.5 + L(mm)/300 |
| Repeatability, RMPE (2s) µm | 1 (Ø: 2) |  |  |  |
| Perpendicularity deviation (frontal), SMPE µm | 5 | 8 | 11 | 25 |
| Max. resolution mm (in) | 0.0001 (0.000005) |  |  |  |
| Measuring force N | 0.75 ÷ 1.5 |  |  |  |
| Autonomy h | 12 |  |  |  |
| Data output | USB / RS232 / Wireless (optional) |  |  |  |
| Air cushion | Yes |  |  |  |
| Weight kg | 22 | 25 | 34 | 41 |

The above values have been determined according to ISO 13225 with the standard measuring insert (TA-MI-101).

Diagram

L: depends on the measuring insert used

STANDARD INSTRUMENT

| The V7 are supplied as follows |  |
| --- | --- |
| Instrument according to specifications | Charging unit (TA-EL-133) |
| Measuring insert with ruby ball Ø 4 mm (TA-MI-101) | Setting gauge (TA-MG-104) |
| Protection cover (TA-TO-114/115/116/117) | User's manual (750 50 0042 03) |
| Calibration certificate |  |

Order numbers

| V7 |  |
| --- | --- |
| V7-400 700 110 10 07 | Measuring range 400 mm |
| V7-700 700 110 20 07 | Measuring range 700 mm |
| V7-1100 700 110 30 07 | Measuring range 1100 mm |
| V7-1800 700 110 50 07 | Measuring range 1800 mm |

applications

| Measurements of height, thickness and chain of dimensions (TA-IH-135, TA-MI-101) | Simultaneous display of diameter and centreline (TA-IH-135, TA-MI-101) | Measurement of perpendicularity with electronic probe (TA-IH-136, TA-MS-101) |
| --- | --- | --- |
| Standard measuring inserts up to 400 mm, with excellent repeatability (TA-IH-131, TA-AD-105, TA-IH-115, V-50.4) | Very large range of accessories for any type of application (TA-SE-106, TA-SE-102, TA-SE-105, TA-SE-107) | Measurement of angles and cones graphically assisted (TA-IH-135, TA-MI-105) |
| Manuel Motorisé |  |  |
| Instant switch from the manual to the motor-driven mode | The motorization guarantees a perfectly constant measuring force (TA-IH-135, TA-MI-101). | 2D measurement extremely simple thanks to the graphical visualisation of the bores |

V8

Introduction

The V8 is the latest addition to the new range of Trimos vertical instruments. The mechanical construction and contents of the packaging are identical to the V9 model with the exception of the handwheel and the display. The V8 shares the characteristics that made the V9 so successful: extreme precision and repeatability, high resolution and Swiss made finish. The specificity of the V8 lies in its handwheel with fine adjustment. It has been specially developed for applications requiring very precise positioning of the key. Like its predecessors, the V8 is distinguished by great ease of use, robust construction and extreme accuracy. The V8's large black mask display offers maximum reading contrast in all light conditions. All functions are directly accessible and represented by easy-to-understand symbols.

Measuring ranges FROM 400 TO 1100 MM

SPECIFIC handwheel WITH FINE ADJUSTMENT

MANUAL MOVEMENT

EXCEPTIONAL PRECISION LEVEL

VAST RANGE OF ACCESSORIES

ALL POSSIBLE ADJUSTMENTS WITHOUT TOOLS

RS232 AND USB INTERFACES

WIRELESS DATA TRANSFER (OPTIONAL)

| Heights |  |
| --- | --- |
| Diameters |  |
| Centreline distances |  |
| Min / Max / Delta |  |
| 9 references |  |
| Perpendicularity |  |
| Average |  |
| Difference |  |
| Temperature compensation |  |
| Wireless communication |  |

dESCRIPTION

Handwheel for carriage displacement and fine adjustment

Cast iron base for optimum stability

Probe holder and Interchangeable measuring insert

Upper probe holder

Floating probe suspension adjustment

"Black Mask" display with intuitive function keys

Handle for instrument displacement with

programmable function keys and air cushion button

Display/Software

Keyboard functions are represented by clear and intuitive symbols. The display on 2 lines offers the user a great working comfort.

EXCELLENT CONTRAST WITH "BLACK MASK" DISPLAY

MEASUREMENTS OF HEIGHTS OR DIAMETERS

MIN / MAX / DELTA MEASURING MODES

ZERO OR PRESET DISPLAY

USB AND RS232 DATA OUTPUT

9 REFERENCES AVAILABLE

GRAPHIC AND SOUND PROBE INDICATOR

Handwheel with fine adjustment specific to V8

| A C B | A Handwheel for manual movement of the carriage B Fine adjustment screw C Locking ring for fine adjustment This handwheel is also available as an option on the V4 and V9. |
| --- | --- |

Technical Specifications

| V8 | 400 | 700 | 1100 |
| --- | --- | --- | --- |
| Measuring range mm (in) | 406 (16) | 710 (28) | 1109 (43) |
| Application range mm (in) | 724 (28) | 1028 (40) | 1427 (56) |
| Max. permissible errors, BMPE µm | 1.2 + L(mm)/1000 |  |  |
| Repeatability, RMPE (2s) µm | 0.4 (Ø: 1) |  |  |
| Perpendicularity deviation (frontal), SMPE µm | 5 | 8 | 11 |
| Max. resolution mm (in) | 0.0001 (0.000005) |  |  |
| Measuring force N | 0.75 ÷ 1.5 |  |  |
| Autonomy h | 12 |  |  |
| Data output | USB / RS232 / Wireless (optional) |  |  |
| Air cushion | Yes |  |  |
| Weight kg | 21 | 24 | 33 |

The above values have been determined according to ISO 13225 with the standard insert (TA-MI-119).

Diagram

L: depends on the measuring insert used

STANDARD INSTRUMENT

| The V8 are supplied as follows |  |
| --- | --- |
| Instrument according to specifications | Charging unit (TA-EL-132) |
| Measuring insert with ruby ball Ø 4 mm (TA-MI-119) | Setting gauge (TA-MG-104) |
| Protection cover (TA-TO-114/115/116) | User's manual (750 50 0045 03) |
| Calibration certificate |  |

Order numbers

| V8 |  |
| --- | --- |
| V8-400 700 110 10 18 | Measuring range 400 mm |
| V8-700 700 110 20 18 | Measuring range 700 mm |
| V8-1100 700 110 30 18 | Measuring range 1100 mm |

applications

| Fine adjustment for specific applications requiring an accurate carriage position | Small diameters measurements with insert Ø 4 mm (V-5, TA-MI-109) | Perpendicularity measurements with electronic probe (TA-IH-126, TA-MS-101) |
| --- | --- | --- |
| Height measurements on watch movement plate (TA-IH-101, TA-MI-115) | Large range of accessories for all types of applications (TA-SE-106, TA-SE-102, TA-SE-105, TA-SE-107) | Very easy data transfer via USB, RS232 or Wireless connection |

|  | SMART REVERSE Diameter measurement faster, more accurate and simpler SmartReverse technology is the result of an intense collaboration between Trimos height gauge users and our R&D team in order to optimize diameter measurements. SmartReverse makes the measurement of diameters very effective by clearly indicating the passage of the reversal points by audible and visual signals. The user is guided precisely during the measurement of diameters, which generates a significant gain in speed and reliability of the measurement. |
| --- | --- |

V9

Introduction

The V9 has been developed for the most demanding users. Laboratories and workshops for which the reliability of the measurement is decisive will appreciate its exceptional level of precision and its "Swiss Made" finish. The metrological performances were at the heart of the development of this height gauge. No compromises on accuracy and repeatability were tolerated. This is why some details of the construction and especially the key holders differ from other models. The display based on a fully tactile interface offers a comfort of use never reached on a height gauge. Menus and functions displayed adhere to a very strict philosophy and design. They thus allow very high work efficiency, even in complex tasks such as programming, 2D mode, angle measurements or statistical analysis of the results. The V9 can be used in manual or motorized mode. The transition from one mode to another can be done at any time with the handwheel.

Measuring ranges FROM 400 TO 1100 MM

EXCEPTIONAL PRECISION LEVEL

ELECTRONICALLY ADJUSTABLE MEASURING FORCE

MANUAL OR MOTORIZED DISPLACEMENT

2D, PROGRAMMING, STATISTICS

VAST RANGE OF ACCESSORIES

ALL POSSIBLE ADJUSTMENTS WITHOUT TOOLS

RS232 AND USB INTERFACES

| Heights |  |
| --- | --- |
| Diameters |  |
| Centreline distances |  |
| Min / Max / Delta |  |
| 9 references |  |
| Perpendicularity |  |
| Average |  |
| Difference |  |
| Angles and cones |  |
| Calculation function |  |
| Tolerances |  |
| 2D mode |  |
| Programs |  |
| Statistics |  |
| Temperature comp. |  |
| USB ports |  |
| RS232 ports |  |
| Wireless communication |  |

dESCRIPTION

Handwheel for carriage displacement (manual/motorized)

Cast iron base for optimum stability

Probe holder and Interchangeable measuring insert

Upper probe holder

Floating probe suspension adjustment

Touch screen display with intuitive function keys

Handle for instrument displacement with

programmable function keys and air cushion button

Display/Software

The choice and position of the symbols as well as the colors used in the interface comply with very strict ergonomic rules. The result is a consistent package that provides readability and maximum user comfort. The Smart Reverse function significantly improves diameter measurements.

VERY SIMPLE GRAPHIC INTERFACE

EXCELLENT readability

MEASUREMENTS IN 2D MODE

MEASUREMENT SEQUENCES

STATISTICAL ANALYSIS OF RESULTS

TEMPERATURE COMPENSATION

SMART REVERSE FUNCTION FOR DIAMETERS

2 coordinates measurements: it has never been so easy

|  | Measuring in 2 coordinates has always been a daunting task on height gauges because the results were not graphically represented. Today, thanks to the live visualization of measurements, it becomes child's play. Just measure the part normally, flip it over and measure it again. The results appear immediately on the screen in 2 dimensions. It is then very easy and instinctive to make measurements by clicking on the different bores measured. |
| --- | --- |

Technical Specifications

| V9 | 400 | 700 | 1100 |
| --- | --- | --- | --- |
| Measuring range mm (in) | 406 (16) | 710 (28) | 1109 (43) |
| Application range mm (in) | 724 (28) | 1028 (40) | 1427 (56) |
| Max. permissible errors, BMPE µm | 1.2 + L(mm)/1000 |  |  |
| Repeatability, RMPE (2s) µm | 0.4 (Ø: 1) |  |  |
| Perpendicularity deviation (frontal), SMPE µm | 5 | 8 | 11 |
| Max. resolution mm (in) | 0.0001 (0.000005) |  |  |
| Measuring force N | 0.75 ÷ 1.5 |  |  |
| Autonomy h | 12 |  |  |
| Data output | USB / RS232 / Wireless (optional) |  |  |
| Air cushion | Yes |  |  |
| Weight kg | 21 | 24 | 33 |

The above values have been determined according to ISO 13225 with the standard insert (TA-MI-119).

SCHEMA

L: depends on the measuring insert used

STANDARD INSTRUMENT

| The V9 are supplied as follows |  |
| --- | --- |
| Instrument according to specifications | Charging unit (TA-EL-133) |
| Measuring insert with ruby ball Ø 4 mm (TA-MI-119) | Setting gauge (TA-MG-104) |
| Protection cover (TA-TO-114/115/116) | User's manual (750 50 0042 03) |
| Calibration certificate |  |

Order numbers

| V9 |  |
| --- | --- |
| V9-400 700 110 10 09 | Measuring range 400 mm |
| V9-700 700 110 20 09 | Measuring range 700 mm |
| V9-1100 700 110 30 09 | Measuring range 1100 mm |

applications

| Height measurements on watch movement plate (TA-MI-115, TA-IH-103) | Small diameters measurements with insert Ø 4 mm (V-50.12, V-5) | Perpendicularity measurements with electronic probe (TA-IH-126, TA-MS-101) |
| --- | --- | --- |
| Very simple measurements in 2 coordinates thanks to the graphic interface | Very wide range of accessories for any type of measurement (TA-SE-102, TA-SE-106, TA-SE-107) | Measurements of angles and cones graphically assisted (TA-MI-101, TA-AD-101) |
| Minimum position measurement thanks to the contour tracking in motorized mode (TA-MI-101, TA-AD-101) | The instrument can be controlled remotely via a PC. | Display adjustable in every direction |

Key Points

CHARACTERISTICS

|  | TVM | V3 | V4 | V5 | V6 | V7 | V8 | V9 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measuring range 300 mm |  |  |  |  |  |  |  |  |
| Measuring range 400 mm |  |  |  |  |  |  |  |  |
| Measuring range 600 mm |  |  |  |  |  |  |  |  |
| Measuring range 700 mm |  |  |  |  |  |  |  |  |
| Measuring range 1100 mm |  |  |  |  |  |  |  |  |
| Measuring range 1800 mm |  |  |  |  |  |  |  |  |
| Precision 20 - 30 µm |  |  |  |  |  |  |  |  |
| Precision 7 - 8 µm |  |  |  |  |  |  |  |  |
| Precision 4.5 - 6 µm |  |  |  |  |  |  |  |  |
| Precision 2.8 µm / 100 mm |  |  |  |  |  |  |  |  |
| Precision 2.2 µm / 100 mm |  |  |  |  |  |  |  |  |
| Precision 1.3 µm / 100 mm |  |  |  |  |  |  |  |  |
| LCD display |  |  |  |  |  |  |  |  |
| "Black Mask" display (1D) |  |  |  |  |  |  |  |  |
| Touch colour display (2D) |  |  |  |  |  |  |  |  |
| Heights, diameters, centrelines |  |  |  |  |  |  |  |  |
| 2D, measuring sequences, statistics |  |  |  |  |  |  |  |  |
| SmartReverse (Smart dia. measurement) |  |  |  |  |  |  |  |  |
| Perpendicularity with lever indicator |  |  |  |  |  |  |  |  |
| Perpendicularity with electronic probe |  |  |  |  |  |  |  |  |
| Manual movement |  |  |  |  |  |  |  |  |
| Manual and motorised movement |  |  |  |  |  |  |  |  |
| Air cushion |  |  |  |  |  |  |  |  |
| Fine adjustment |  |  |  |  |  |  |  |  |
| Long probes 400 mm |  |  |  |  |  |  |  |  |
| Wireless data transfer (Option) |  |  |  |  |  |  |  |  |

| 1D Display | V3, V4, V5, V6 and V8 models: A 1D height gauge offers basic features such as height, diameter, centerline distance, min / max measurements. It is very appreciated in workshop for its great simplicity of use. Keyboard functions are represented by clear and intuitive symbols. The 2-line screen offers the user a great working comfort. - Easy to read at any time thanks to the "Black Mask" display - Measurement of heights or diameters - MIN / MAX / DELTA measurement modes - Resetting or presetting the display - USB and RS232 interfaces - 9 references available - Graphic and acoustic indicator |
| --- | --- |
| 2D Display | V7 and V9 models: A 2D height gauge offers extensive measuring possibilities with a very high level of user comfort. Thus, the measurements in 2 coordinates and the programming of the measurement sequences can be performed very efficiently. The touch screen and the graphical interface correspond to the most modern industrial standards. The great flexibility offered by the touch screen makes it easy and quick to use. - Very simple graphical interface - Exceptional readability - 2D mode measurement - Measurement sequences - Statistical analysis of the results - Integrated online help - Temperature compensation |
| Long Probes | The robust construction of the Trimos height gauges allows the use of very diverse inserts up to 400 mm in length with breathtaking repeatability. |

| SMART REVERSE | In order to provide exceptional measurement performance, the Trimos team has collaborated with many users to develop the SMART REVERSE technology. The result is incredible and opens a new dimension to the measurement of diameters. All the visual, auditory and tactile senses of the user are exploited to provide an unrivalled measurement experience in terms of comfort, speed and accuracy. |
| --- | --- |
| MANUAL AND MOTORIZED MOVEMENT Manual Motorized | The V5, V6, V7 and V9 are equipped with a revolutionary handwheel which allows the user to move the measuring carriage either manually or motorized. Each of these modes does not suffer from any compromise, that is to say that the user preferring a manual instrument will not notice any difference compared to a conventional manual instrument, ditto for a motorized movement. This innovation avoids making a difficult choice during the acquisition and makes it possible to satisfy the multiple potential users of the same instrument. The other models are fully manual. |
| Wireless communication | In addition to data transfer capabilities via RS232 or USB, Trimos offers a perfectly integrated wireless communication solution. The free software TrimosDataTransfer makes it possible to recover the data in any application. It is also compatible with Sylvac software solutions Vmux and Sylcom. This option is available on all models except V3 and TVM. |

| Professional Test Reports | Professional test reports can be generated directly from the instrument (V7 and V9). The interface allows you to create a personalized header template with company logo. The drawing of the part can also be integrated into the report. All measured dimensions are shown with their tolerance in graphical form. This makes reading the results very easy and efficient. The report can be printed directly on any USB printer or in pdf format on a USB key. |
| --- | --- |
| Ergonomics | With a left-hand movement handle and a right-hand handwheel, the user's position is at all times optimal relative to the measured part regardless of size and shape. The user keeps control of the instrument and the movement of the piece at all times. The orientation of the screen ensures perfect readability in all configurations. |
| Accessories | The range of available accessories covers almost all applications. The unique key weight compensation system also allows the use of specific keys up to 400 g. |
| Tradition and SWISS MADE | 45 years ago, Trimos was the first manufacturer in the world to launch a height gauge on the market. Since then, the company has constantly offered its customers innovative and efficient products, always ahead of its competitors. Our values of excellence, precision and quality are rooted in the Swiss tradition. All our height gauges are developed and manufactured in Switzerland. That's why they wear the Swiss Made label. |

ACCESSORiES